	PA28-181  FLIGHT  CHECKLIST

N43839
BEFORE  STARTING  ENGINE
  1.  Seats, Belts & Shoulder Harness  --  CHECK and ADJUST

  2.  Brakes --  ON

  3.  Fuel Selector  --  FULLEST  TANK

  4.  Circuit Breakers  --  CHECK  IN

  5.  Radios, Electrical Switches  --  OFF

STARTING  ENGINE
  1.  Carburetor Heat  --  OFF

  2.  Mixture  ---  RICH

  3.  Throttle  --  OPEN 1/4 inch

  4.  Master Switch  --  ON

  5.  Primer  --  PRIME  AS  REQUIRED

  6.  Fuel Pump  --  ON

  7.  Propeller Area --  CLEAR

  8.  Ignition Switch  --  START

  9.  Throttle  --  800  to  1000  RPM

10.  Oil Pressure  --  CHECK

11.  Fuel Pump  --  OFF

12.  Flaps --  UP

13.  Radios  --  ON

14.  Rotating Beacon, Strobes  --  ON

15.  Transponder  --   STANDBY

BEFORE-TAKEOFF
  1.  Cabin Door  -- CLOSED and LATCHED

  2.  Flight Controls  --  CHECK

  3.  Trim  --  CHECK OPERATION/TAKEOFF SETTING

  4.  Throttle  --  1700 RPM

  5.  Mixture  --  ADJUST (Rich below 5000' DENSITY ALTITUDE)

  6.  Engine Instruments  --  GREEN  ARC

  7.  Vacuum Gage  --  4.6 to 5.4 Hg

  8.  Magnetos  --  CHECK (150 RPM Max. drop, 75 RPM max. diff)

  9.  Carburetor Heat  --  CHECK  OPERATION  THEN  "OFF"

10.  Annunciator Panel  --  PRESS  TO  TEST


	
	11. Throttle  --   800  to 1000 RPM

12.  Flight Instruments  --  SET (Heading , Attitude & Altimeter)

13.  Radios  --  SET

14.  Transponder  --  ALT

15.  Fuel Pump  --  ON

16.  Flaps  --  0 to 25 Degrees

TAKE-OFF
  1.  Climb Airspeed  --  Vy 76 KTS   Vx  64 KTS

  2.  Flaps  --  UP

CRUISE  CLIMB
  1.  Airspeed  --  90 KTS

  2.  Fuel Pump  --  OFF

BEFORE  LANDING
  1.  Carburetor Heat  --  ON

  2.  Mixture  --  RICH

  3.  Fuel Pump  --  ON

  4.  Fuel Selector  --  FULLEST  TANK

  5.  Flaps  -- AS  DESIRED  BELOW  103 KTS

  6.  Airspeed  --  65 to 70 KTS (flaps down)   Best Glide (76 KTS) 

AFTER LANDING
  1.  Carburetor Heat  --  OFF

  2.  Flaps  --  UP

  3.  Fuel Pump  --  OFF 

4.  Transponder – STANDBY

SECURING AIRCRAFT
  1.  Radios  --  OFF

  2.  Electrical Equipment  --  OFF

  3.  Mixture  --  IDLE-CUT OFF

  4.  Ignition Switch  --  OFF

  5.  Master Switch  --  OFF

  6.  Control Wheel  --  SECURED  WITH  SEAT BELT

  7.  Record Time 

8.  CLOSE  FLIGHT  PLAN

9.  Lock Aircraft and Tiedown




